Supplementary materials for:

A rapid hazard-to-network framework for assessing earthquake-induced transport disruption and road-network connectivity

Information

The Supplementary Materials provide the observational constraints and parameter sets that support the three physical modules of the event-conditioned hazard-to-network framework described in the main article. Appendix A complements Sections 2.2 and 3.2 by documenting the ground-motion observations used to evaluate the GMPE-derived PGA and Iₐ fields. Appendix B complements Sections 2.3 and 3.3 by presenting the bridge fragility and functional-interruption parameters. Appendix C complements Sections 2.4 and 3.4 by documenting the lithology-based geotechnical inputs used in the Newmark displacement and road-blockage analyses. Tables S1-S3 are placed within their corresponding appendices so that each parameter set remains directly connected to its analytical role.

Appendix A. Earthquake ground-motion parameters and corresponding analysis

The ground-motion module uses the 2025 Mw 7.7 Sagaing earthquake source description, including moment magnitude, finite-fault geometry, rupture-distance metrics, focal mechanism, and Vs30-based site conditions (Heath et al. 2020), to construct the lower, median, and upper CB19 scenarios. PGA characterizes shaking amplitude and supplies the bridge-fragility demand, whereas Iₐ represents cumulative shaking energy and is paired with PGA for Newmark displacement modelling. Regional strong-motion records, named-location observations, UTM-referenced intensity points, and the USGS ensemble provide the observational basis for the attenuation comparison in Fig. 4a of the main article. The same scenario index is propagated through the bridge, landslide, and network modules.

Table S1 compiles the spatially referenced PGA, PGV, rupture-distance, MMI, and location information used to evaluate the attenuation and regional consistency of the adopted ground-motion fields. The named entries and UTM-coded observation points extend the comparison from the near-fault corridor to the far field and support the ground-motion interpretation reported in Section 3.2.

Table S1. Station data for GMPE validation

| Station | PGA(g) | PGV(cm/s) | Distance(km) | mmi | Location | Intensity |
| --- | --- | --- | --- | --- | --- | --- |
| NPW, Naypyitaw, Myanmar | 23.77% | 62.92 | 2.75 | VIII | 19.779°N 96.138°E | 8.2 |
| UTM:(47Q 0199 2185 1000) | 91.76% | 64.77 | 3.83 | VIII | 19.742°N 96.133°E | 8.2 |
| UTM:(47Q 0198 2432 1000) | 125.25% | 93.24 | 9.85 | IX | 21.971°N 96.081°E | 8.7 |
| UTM:(47Q 0198 2435 1000) | 63.17% | 41.83 | 9.89 | VIII | 21.998°N 96.080°E | 7.6 |
| UTM:(47Q 0199 2432 1000) | 115.08% | 86.69 | 10.85 | IX | 21.971°N 96.090°E | 8.6 |
| UTM:(47Q 0200 2431 1000) | 105.41% | 80.6 | 11.9 | IX | 21.962°N 96.100°E | 8.5 |
| UTM:(47Q 0237 2438 1000) | 49.14% | 52.05 | 48.95 | VIII | 22.032°N 96.457°E | 7.9 |
| NGU, Ngaung U, Myanmar | 5.65% | 8.95 | 112.94 | VI | 21.206°N 94.917°E | 5.5 |
| UTM:(47Q 0206 1866 1000) | 4.04% | 4.06 | 155.78 | IV | 16.864°N 96.246°E | 4.4 |
| YGN, Yangon, Myanmar | 2.43% | 10.46 | 157.14 | VI | 16.865°N 96.153°E | 5.7 |
| UTM:(47Q 0198 1865 1000) | 7.05% | 7.83 | 158.06 | V | 16.853°N 96.171°E | 5.3 |
| UTM:(47Q 0193 1864 1000) | 5.83% | 6.29 | 160.04 | V | 16.844°N 96.124°E | 5 |
| UTM:(47Q 0202 1862 1000) | 4.54% | 4.7 | 160.32 | V | 16.827°N 96.209°E | 4.6 |
| UTM:(47Q 0194 1863 1000) | 4.54% | 4.7 | 160.81 | V | 16.835°N 96.133°E | 4.6 |
| UTM:(47Q 0193 1863 1000) | 5.47% | 5.85 | 161.02 | V | 16.835°N 96.124°E | 4.9 |
| UTM:(47Q 0196 1860 1000) | 5.45% | 5.85 | 163.35 | V | 16.808°N 96.153°E | 4.9 |
| UTM:(47Q 0193 1858 1000) | 6.54% | 7.28 | 165.9 | V | 16.790°N 96.125°E | 5.2 |
| UTM:(47Q 0196 1857 1000) | 3.97% | 4.06 | 166.29 | IV | 16.781°N 96.153°E | 4.4 |
| UTM:(47Q 0197 1856 1000) | 3.62% | 3.78 | 167.09 | IV | 16.772°N 96.163°E | 4.3 |
| UTM:(47Q 0373 2537 1000) | 24.73% | 36.16 | 191 | VII | 22.941°N 97.766°E | 7.4 |
| UTM:(47Q 0441 2140 1000) | 6.36% | 7.83 | 229.68 | V | 19.358°N 98.443°E | 5.3 |
| UTM:(47Q 0491 2066 1000) | 2.66% | 3.78 | 267.4 | IV | 18.690°N 98.919°E | 4.3 |
| CHTO, Chiang Mai, Thailand | 1.13% | 2.68 | 272.22 | IV | 18.814°N 98.944°E | 4 |
| UTM:(47Q 0494 2079 1000) | 3.05% | 4.06 | 272.46 | IV | 18.807°N 98.948°E | 4.4 |
| UTM:(47Q 0495 2079 1000) | 5.03% | 6.29 | 273.45 | V | 18.807°N 98.957°E | 5 |
| UTM:(47Q 0496 2078 1000) | 4.44% | 5.44 | 274.28 | V | 18.798°N 98.967°E | 4.8 |
| UTM:(47Q 0497 2075 1000) | 6.06% | 7.83 | 274.78 | V | 18.771°N 98.976°E | 5.3 |
| UTM:(47Q 0496 2082 1000) | 3.03% | 4.06 | 274.92 | IV | 18.834°N 98.967°E | 4.4 |
| UTM:(47Q 0497 2077 1000) | 1.94% | 3.16 | 275.1 | IV | 18.789°N 98.976°E | 4.1 |
| UTM:(47Q 0498 2075 1000) | 4.16% | 5.06 | 275.77 | V | 18.771°N 98.986°E | 4.7 |
| UTM:(47Q 0498 2076 1000) | 11.27% | 16.22 | 275.93 | VI | 18.780°N 98.986°E | 6.3 |
| UTM:(47Q 0498 2077 1000) | 6.05% | 7.83 | 276.09 | V | 18.789°N 98.986°E | 5.3 |
| UTM:(47Q 0498 2078 1000) | 3.50% | 4.37 | 276.25 | V | 18.798°N 98.986°E | 4.5 |
| UTM:(47Q 0499 2074 1000) | 13.58% | 20.19 | 276.59 | VII | 18.762°N 98.995°E | 6.6 |
| UTM:(47Q 0499 2075 1000) | 9.34% | 13.04 | 276.75 | VI | 18.771°N 98.995°E | 6 |
| UTM:(47Q 0499 2076 1000) | 5.34% | 6.77 | 276.91 | V | 18.780°N 98.995°E | 5.1 |
| UTM:(47Q 0499 2077 1000) | 3.91% | 4.7 | 277.08 | V | 18.789°N 98.995°E | 4.6 |
| UTM:(47Q 0499 2078 1000) | 7.28% | 9.74 | 277.24 | VI | 18.798°N 98.995°E | 5.6 |
| UTM:(47Q 0500 2074 1000) | 10.57% | 15.08 | 277.57 | VI | 18.762°N 99.005°E | 6.2 |
| UTM:(47Q 0500 2075 1000) | 10.57% | 15.08 | 277.73 | VI | 18.771°N 99.005°E | 6.2 |
| UTM:(47Q 0500 2076 1000) | 9.93% | 14.02 | 277.89 | VI | 18.780°N 99.005°E | 6.1 |
| UTM:(47Q 0501 2079 1000) | 8.76% | 12.12 | 279.36 | VI | 18.807°N 99.014°E | 5.9 |
| UTM:(47Q 0502 2078 1000) | 5.66% | 7.28 | 280.19 | V | 18.798°N 99.024°E | 5.2 |
| UTM:(47Q 0503 2078 1000) | 5.31% | 6.77 | 281.18 | V | 18.798°N 99.033°E | 5.1 |
| UTM:(47Q 0454 1847 1000) | 5.99% | 7.83 | 285.22 | V | 16.710°N 98.573°E | 5.3 |
| UTM:(47Q 0557 2018 1000) | 2.12% | 3.51 | 329.46 | IV | 18.255°N 99.544°E | 4.2 |
| KTN Keng Tun, Shan, Myanmar | 1.38% | 3.82 | 371.91 | IV | 21.286°N 99.590°E | 4.3 |
| UTM:(47Q 0634 1860 1000) | 5.91% | 7.83 | 436.79 | V | 16.824°N 100.262°E | 5.3 |
| UTM:(47P 0716 1672 1000) | 3.82% | 4.7 | 599.41 | V | 15.119°N 101.015°E | 4.6 |
| UTM:(47P 0664 1550 1000) | 4.07% | 5.06 | 641.03 | V | 14.020°N 100.523°E | 4.7 |
| UTM:(47P 0673 1556 1000) | 2.46% | 3.78 | 642.82 | IV | 14.074°N 100.607°E | 4.3 |
| UTM:(47P 0661 1538 1000) | 1.58% | 2.7 | 647.84 | IV | 13.912°N 100.495°E | 4 |
| UTM:(47P 0664 1536 1000) | 4.07% | 5.06 | 651.33 | V | 13.894°N 100.522°E | 4.7 |
| UTM:(47P 0671 1537 1000) | 2.46% | 3.78 | 655.28 | IV | 13.902°N 100.587°E | 4.3 |
| UTM:(47P 0657 1523 1000) | 6.69% | 9.06 | 656.43 | VI | 13.777°N 100.457°E | 5.5 |
| UTM:(47P 0663 1528 1000) | 4.33% | 5.44 | 656.61 | V | 13.822°N 100.513°E | 4.8 |
| UTM:(47P 0669 1531 1000) | 1.83% | 3.16 | 658.37 | IV | 13.848°N 100.568°E | 4.1 |
| UTM:(47P 0658 1521 1000) | 4.90% | 6.29 | 658.6 | V | 13.759°N 100.466°E | 5 |
| UTM:(47P 0664 1526 1000) | 5.22% | 6.77 | 658.77 | V | 13.803°N 100.522°E | 5.1 |
| UTM:(47P 0658 1520 1000) | 3.32% | 4.37 | 659.35 | V | 13.750°N 100.466°E | 4.5 |
| UTM:(47P 0670 1530 1000) | 4.61% | 5.85 | 659.78 | V | 13.839°N 100.578°E | 4.9 |
| UTM:(47P 0669 1529 1000) | 4.61% | 5.85 | 659.85 | V | 13.830°N 100.568°E | 4.9 |
| UTM:(47P 0660 1521 1000) | 4.33% | 5.44 | 659.89 | V | 13.758°N 100.485°E | 4.8 |
| UTM:(47P 0669 1528 1000) | 7.58% | 10.48 | 660.6 | VI | 13.821°N 100.568°E | 5.7 |
| UTM:(47P 0668 1527 1000) | 5.55% | 7.28 | 660.67 | V | 13.812°N 100.559°E | 5.2 |
| UTM:(47P 0666 1525 1000) | 1.58% | 2.7 | 660.84 | IV | 13.794°N 100.540°E | 4 |
| UTM:(47P 0662 1521 1000) | 2.86% | 4.06 | 661.2 | IV | 13.758°N 100.503°E | 4.4 |
| UTM:(47P 0662 1520 1000) | 2.12% | 3.51 | 661.96 | IV | 13.749°N 100.503°E | 4.2 |
| UTM:(47P 0669 1526 1000) | 4.90% | 6.29 | 662.08 | V | 13.803°N 100.568°E | 5 |
| UTM:(47P 0666 1523 1000) | 4.33% | 5.44 | 662.33 | V | 13.776°N 100.540°E | 4.8 |
| UTM:(47P 0663 1520 1000) | 5.55% | 7.28 | 662.61 | V | 13.749°N 100.512°E | 5.2 |
| UTM:(47P 0659 1516 1000) | 4.61% | 5.85 | 663.03 | V | 13.713°N 100.475°E | 4.9 |
| UTM:(47P 0665 1521 1000) | 4.07% | 5.06 | 663.17 | V | 13.758°N 100.531°E | 4.7 |
| UTM:(47P 0666 1521 1000) | 4.07% | 5.06 | 663.83 | V | 13.758°N 100.540°E | 4.7 |
| UTM:(47P 0665 1520 1000) | 2.86% | 4.06 | 663.92 | IV | 13.749°N 100.531°E | 4.4 |
| UTM:(47P 0663 1518 1000) | 6.29% | 8.42 | 664.12 | V | 13.731°N 100.512°E | 5.4 |
| UTM:(47P 0668 1522 1000) | 3.32% | 4.37 | 664.4 | V | 13.767°N 100.559°E | 4.5 |
| UTM:(47P 0676 1529 1000) | 5.22% | 6.77 | 664.53 | V | 13.830°N 100.633°E | 5.1 |
| UTM:(47P 0666 1520 1000) | 5.55% | 7.28 | 664.58 | V | 13.749°N 100.540°E | 5.2 |
| UTM:(47P 0665 1519 1000) | 4.07% | 5.06 | 664.68 | V | 13.740°N 100.531°E | 4.7 |
| UTM:(47P 0664 1518 1000) | 3.82% | 4.7 | 664.78 | V | 13.731°N 100.521°E | 4.6 |
| UTM:(47P 0663 1517 1000) | 4.07% | 5.06 | 664.87 | V | 13.722°N 100.512°E | 4.7 |
| UTM:(47P 0670 1523 1000) | 6.29% | 8.42 | 664.97 | V | 13.776°N 100.577°E | 5.4 |
| UTM:(47P 0669 1522 1000) | 5.55% | 7.28 | 665.06 | V | 13.767°N 100.568°E | 5.2 |
| UTM:(47P 0666 1519 1000) | 3.32% | 4.37 | 665.33 | V | 13.740°N 100.540°E | 4.5 |
| UTM:(47P 0665 1518 1000) | 4.61% | 5.85 | 665.43 | V | 13.731°N 100.531°E | 4.9 |
| UTM:(47P 0664 1517 1000) | 3.32% | 4.37 | 665.53 | V | 13.722°N 100.521°E | 4.5 |
| UTM:(47P 0663 1516 1000) | 4.33% | 5.44 | 665.63 | V | 13.713°N 100.512°E | 4.8 |
| UTM:(47P 0655 1509 1000) | 3.82% | 4.7 | 665.78 | V | 13.650°N 100.438°E | 4.6 |
| UTM:(47P 0669 1521 1000) | 5.91% | 7.83 | 665.81 | V | 13.758°N 100.568°E | 5.3 |
| UTM:(47P 0668 1520 1000) | 5.91% | 7.83 | 665.9 | V | 13.749°N 100.559°E | 5.3 |
| UTM:(47P 0667 1519 1000) | 4.61% | 5.85 | 665.99 | V | 13.740°N 100.549°E | 4.9 |
| UTM:(47P 0666 1518 1000) | 6.29% | 8.42 | 666.09 | V | 13.731°N 100.540°E | 5.4 |
| UTM:(47P 0665 1517 1000) | 5.22% | 6.77 | 666.18 | V | 13.722°N 100.531°E | 5.1 |
| UTM:(47P 0669 1520 1000) | 4.90% | 6.29 | 666.56 | V | 13.749°N 100.568°E | 5 |
| UTM:(47P 0668 1519 1000) | 5.22% | 6.77 | 666.64 | V | 13.740°N 100.558°E | 5.1 |
| UTM:(47P 0667 1518 1000) | 4.61% | 5.85 | 666.74 | V | 13.731°N 100.549°E | 4.9 |
| UTM:(47P 0669 1519 1000) | 4.61% | 5.85 | 667.31 | V | 13.740°N 100.568°E | 4.9 |
| UTM:(47P 0668 1518 1000) | 3.82% | 4.7 | 667.39 | V | 13.731°N 100.558°E | 4.6 |
| UTM:(47P 0684 1532 1000) | 3.82% | 4.7 | 667.75 | V | 13.856°N 100.707°E | 4.6 |
| UTM:(47P 0669 1518 1000) | 2.12% | 3.51 | 668.05 | IV | 13.731°N 100.568°E | 4.2 |
| UTM:(47P 0668 1517 1000) | 4.90% | 6.29 | 668.14 | V | 13.722°N 100.558°E | 5 |
| UTM:(47P 0670 1518 1000) | 5.55% | 7.28 | 668.71 | V | 13.731°N 100.577°E | 5.2 |
| UTM:(47P 0671 1518 1000) | 4.90% | 6.29 | 669.38 | V | 13.731°N 100.586°E | 5 |
| UTM:(47P 0670 1517 1000) | 2.86% | 4.06 | 669.46 | IV | 13.722°N 100.577°E | 4.4 |
| UTM:(47P 0661 1509 1000) | 1.58% | 2.7 | 669.63 | IV | 13.650°N 100.493°E | 4 |
| UTM:(47P 0671 1517 1000) | 1.83% | 3.16 | 670.12 | IV | 13.722°N 100.586°E | 4.1 |
| UTM:(47P 0671 1516 1000) | 3.82% | 4.7 | 670.87 | V | 13.713°N 100.586°E | 4.6 |
| UTM:(47P 0672 1516 1000) | 2.46% | 3.78 | 671.53 | IV | 13.713°N 100.595°E | 4.3 |
| UTM:(47P 0673 1516 1000) | 4.33% | 5.44 | 672.19 | V | 13.712°N 100.605°E | 4.8 |
| UTM:(47P 0672 1515 1000) | 3.32% | 4.37 | 672.27 | V | 13.704°N 100.595°E | 4.5 |
| UTM:(47P 0673 1515 1000) | 4.33% | 5.44 | 672.93 | V | 13.703°N 100.604°E | 4.8 |
| UTM:(47P 0680 1521 1000) | 5.91% | 7.83 | 673.12 | V | 13.757°N 100.670°E | 5.3 |
| UTM:(47P 0689 1529 1000) | 5.55% | 7.28 | 673.34 | V | 13.829°N 100.753°E | 5.2 |
| UTM:(47P 0673 1514 1000) | 4.61% | 5.85 | 673.68 | V | 13.694°N 100.604°E | 4.9 |
| UTM:(47P 0674 1513 1000) | 4.90% | 6.29 | 675.09 | V | 13.685°N 100.614°E | 5 |
| UTM:(47P 0679 1516 1000) | 2.12% | 3.51 | 676.16 | IV | 13.712°N 100.660°E | 4.2 |
| UTM:(47P 0678 1513 1000) | 3.32% | 4.37 | 677.73 | V | 13.685°N 100.651°E | 4.5 |
| UTM:(47P 0676 1511 1000) | 3.32% | 4.37 | 677.9 | V | 13.667°N 100.632°E | 4.5 |
| UTM:(47P 0675 1510 1000) | 3.32% | 4.37 | 678 | V | 13.658°N 100.623°E | 4.5 |
| UTM:(47P 0689 1514 1000) | 4.07% | 5.06 | 684.32 | V | 13.693°N 100.752°E | 4.7 |

Together with the station-derived Iₐ values reported in Section 3.2, the Table S1 observations constrain the paired PGA-Iₐ scenario fields. PGA is sampled at bridge locations, while paired PGA-Iₐ fields are resampled to the critical-acceleration grid for Newmark modelling; this common scenario basis maintains consistency across the bridge-only, landslide-only, and compound road-network assessments.

Appendix B. Bridge parameters and corresponding analysis

The bridge module in Sections 2.3 and 3.3 converts scenario-specific PGA at bridge locations into damage-state exceedance probabilities using lognormal fragility functions. The available Myanmar bridge inventory is associated with the processed road graph and matched to a representative HAZUS highway-bridge class. Slight, moderate, extensive, and complete damage states describe the progression of structural response, while extensive damage provides the closure-level state used to represent functional interruption.

Table S2 summarizes the median PGA thresholds θds, total lognormal dispersion βds, closure-level state DScl, and interruption threshold TBR used in Eqs. (3)-(5) and (11) of the main article. These parameters connect the ground-motion field to bridge-level and road-segment interruption states.

Table S2. HAZUS-referenced bridge fragility parameters used for Myanmar bridge-interruption assessment

| Parameter | Damage state / criterion | Adopted value | Use in this study | Basis |
| --- | --- | --- | --- | --- |
| Reference bridge class | Ordinary Myanmar road/highway bridges matched to a representative HAZUS highway-bridge class | — | Defines the bridge fragility parameter set | Myanmar bridge inventory and HAZUS bridge classes |
| Median PGA, θ | Slight | 0.12 g | Slight-damage exceedance | HAZUS-type bridge fragility parameter |
| Median PGA, θ | Moderate | 0.17 g | Moderate-damage exceedance | HAZUS-type bridge fragility parameter |
| Median PGA, θ | Extensive | 0.32 g | Closure-level damage state | HAZUS-type bridge fragility parameter |
| Median PGA, θ | Complete | 0.54 g | Complete-damage exceedance | HAZUS-type bridge fragility parameter |
| Lognormal dispersion, β | All damage states | 0.64 | Fragility-curve dispersion | HAZUS-type bridge fragility parameter |
| Closure-level state | DScl | Extensive | Bridge functional-interruption assessment | Emergency accessibility screening |
| Interruption threshold | TBR | 0.60 | Binary bridge / road-segment interruption | Network screening criterion |

Note: The bridge fragility parameters were selected by matching the available Myanmar bridge inventory to the closest representative HAZUS highway-bridge class (FEMA 2024; Council 2020). The damage-state structure, median PGA values, and lognormal dispersion follow the HAZUS earthquake bridge fragility framework, which is widely used for regional earthquake loss and infrastructure-disruption assessment. Extensive damage was adopted as the closure-level state to represent bridge functional interruption in emergency accessibility analysis. The threshold τBR=0.60converts the scenario-conditioned bridge-disruption indicator into a binary road-segment interruption state for network modelling.

The resulting bridge-related segment states form the bridge-only disruption scenario and one pathway of the compound scenario described in Section 2.5. Their spatial concentration and emergency-route effects are reported in Sections 3.3 and 3.5.

Appendix C. Geological parameters and corresponding coseismic-landslide analysis

The landslide module in Sections 2.4 and 3.4 combines DEM-derived slope with lithology-conditioned cohesion c, internal friction angle φ, natural unit weight γ, and sliding thickness. These inputs determine the horizontal critical-acceleration coefficient in Eq. (6). Scenario-consistent PGA and Iₐ fields are then applied in the Newmark displacement model, and cells in the high and very high classes (DN > 5 cm) delineate landslide-prone areas for road-overlay analysis.

Table S3 groups the Myanmar geological codes into five engineering geomaterial classes and reports parameter ranges together with the representative values used in the regional calculation. In each range/representative entry, the value following the slash is the model input used to generate the parameter fields and the scenario-specific landslide classes.

Table S3.  Lithology-based equivalent geotechnical parameters for regional Newmark modelling in Myanmar.

| Class | Engineering geomaterial class | Mapped codes | Main lithology | Internal Friction Angle φ (°) | Cohesion c (kPa) | Natural Unit Weight γ (kN/m³) | Basis |
| --- | --- | --- | --- | --- | --- | --- | --- |
| I | Unconsolidated alluvial and deltaic deposits | Q | alluvial clay, silt, sand, gravel, deltaic deposits | 18–32 / 24 | 0–40 / 12 | 17–20 / 18.5 | Yangon–Thilawa soft clay, Yangon River alluvial–deltaic deposits, JICA port-area soil investigation |
| II | Weathered soft sedimentary rocks and residual soils | N / Mi | mudstone, shale, weak sandstone, residual soil | 20–34 / 26 | 5–45 / 18 | 18–21 / 19.2 | weak sedimentary and residual-soil units |
| III | Weak sedimentary rock mass and highly weathered sandstone–shale slopes | Pg / Ts / Ks / S / Osm | sandstone, shale, siltstone, weak limestone, flysch-like sediments | 24–36 / 30 | 10–60 / 28 | 19–22.5 / 20.8 | weathered bedding-controlled sedimentary slopes |
| IV | Moderately weathered sedimentary and metasedimentary rock mass | Cs / Jms / TrCs / Pz / PTr / Pr / J / Pzi / Pzl / Trms / Tms | sandstone, limestone, schist, slate, phyllite, marble | 28–40 / 34 | 20–80 / 45 | 21–25 / 23.0 | rock-mass equivalent parameters |
| V | Hard crystalline, volcanic, and carbonate rock mass | CMi / Cmsm / I / Ji / Ki / Pzu / Pzui / TKim / Ti / Tv / V / pC | granite, basalt, gneiss, migmatite, massive limestone, volcanic rocks | 32–44 / 38 | 30–100 / 65 | 24–27 / 25.5 | discontinuity-controlled hard-rock slopes |
| — | Excluded class | H2O / H20 / blank GLG | water bodies or unclassified pixels | — | — | — | NoData |

Note: The parameters are Myanmar-informed equivalent Mohr–Coulomb values for regional Newmark landslide screening, not site-specific design strengths or intact-rock properties. Geological codes were grouped into five engineering geomaterial classes according to Myanmar’s regional geological context. Parameter ranges were constrained by Myanmar geotechnical investigation requirements, Yangon–Thilawa soil investigation data, published Yangon River soft-clay properties, and rock-mass equivalent strength concepts (Yamada et al. 2018; Agency et al. 2014; Agency et al. 2015; Wandrey and Law 1998; Construction 2020; Hoek 2007; Hoek et al. 2002; Yamada et al. 2023). Representative values were adopted for rapid scenario-based seismic landslide and transport-network disruption assessment with a 3 m shallow sliding thickness. Water bodies and blank geological codes were assigned NoData. Site-specific field and laboratory calibration is required for engineering design.

The resulting critical-acceleration and Newmark-displacement rasters are intersected with road segments and exposure buffers. Road segments intersecting high or very high cells receive the landslide-blockage state used in the landslide-only and compound network scenarios.

Supplementary References

Agency, Japan International Cooperation, The Overseas Coastal Area Development Institute of Japan, and Ltd. Nippon Koei Co. 2014. The Preparatory Survey for the Project for Expansion of Yangon Port in Thilawa Area: Final Report 1. Tokyo: Japan International Cooperation Agency.

Agency, Japan International Cooperation, Ltd. Kamigumi Co., and Toyota Tsusho Corporation. 2015. Preparatory Survey on Yangon Port in Thilawa Area and Logistics Depot Development in the Republic of the Union of Myanmar: Final Report. Tokyo: Japan International Cooperation Agency.

Construction, The Republic of the Union of Myanmar Ministry of. 2020. Myanmar National Building Code (MNBC). In Part 4: Soils And Foundations: International Relation and Legal Section Department of Building Ministry of Construction

Council, Myanmar Engineering. 2020. Myanmar National Building Code 2020: Part 3 Structural Design and Part 4 Soil and Foundation. Yangon, Myanmar: Myanmar Engineering Council.

FEMA. 2024. HAZUS-MH 6.1 Earthquake Model Technical Manual.

Heath, David C., David J. Wald, C. Bruce Worden, Eric M. Thompson, and Gregory M. Smoczyk. 2020. A global hybrid VS30 map with a topographic slope–based default and regional map insets. Earthquake Spectra 36 (3):1570-1584.

Hoek, Evert. 2007. Rock Mass Properties. In Practical Rock Engineering. Vancouver: Rocscience.

Hoek, Evert, Carlos Carranza-Torres, and Brent Corkum. 2002. Hoek-Brown Failure Criterion: 2002 Edition. In Proceedings of the 5th North American Rock Mechanics Symposium and the 17th Tunnelling Association of Canada Conference. Toronto, Canada: University of Toronto Press.

Wandrey, Craig J., and Ben E. Law. 1998. Maps showing geology, oil and gas fields and geologic provinces of South Asia. In Open-File Report. Reston, VA.

Yamada, Yoshimitsu, Takashi Tsuchida, Nyan Myint Kyaw, Tatsuru Aoyama, Toshiya Akasaki, Moe Myint Su Hlaing, and Ryota Hashimoto. 2023. Geotechnical properties of soft clays along Yangon River – Characteristics of soft clays in Thilawa Port area. Soils and Foundations 63 (4):101331.

Yamada, Yoshimitsu, Takashi Tsuchida, Nyan Myint Kyaw, Tatsuru Aoyama, Genki Takahashi, Si Thu Aung, and Moe Myint Su Hlaing. 2018. Physical and Mechanical Properties of Soft Clay Distributed at Thilawa Area along Yangon River Based on Soil Investigation Results Carried Out under JGS Standard. Japanese Geotechnical Journal 13 (1):85-99.